PROJECT REQUIREMENT DOCUMENT

AetherCodec

Open-Source Hi-Res Bluetooth Audio Codec

| Version | 1.0 — Initial Release |
| --- | --- |
| Date | July 2026 |
| Status | Draft — Internal |
| Project Type | Open Source / Research / CS Student Project |
| Budget | ₹0 (Zero Budget) |
| Target Stack | C Language, Linux (PipeWire/ALSA), Windows (WASAPI) |
| Test Hardware | 2 Laptops with Bluetooth + 3.5mm headphone jack |

Confidential — For Internal Review Only

# 1. Executive Summary

AetherCodec is a student-led open-source project to design, implement, and demonstrate a novel Bluetooth audio codec capable of streaming near-lossless and high-quality perceptual audio at resolutions up to 24-bit/96kHz — surpassing the effective quality ceiling of currently available commercial codecs such as LDAC, aptX HD, and Sony's SSC.

The project is motivated by a clear gap in the market: no existing Bluetooth audio codec is open-source, Linux-native, and lossless-first. AetherCodec aims to fill this gap with a transparent, documented, and community-extensible architecture.

| Near-Lossless Mode | Perceptual HQ Mode | Adaptive Bitrate | Linux + Windows | 100% Open Source |
| --- | --- | --- | --- | --- |

# 2. Problem Statement

## 2.1 The Bluetooth Audio Quality Gap

Bluetooth audio has fundamentally stagnated at lossy compression. Every commercially available Bluetooth audio codec — regardless of marketing claims — discards audio information during encoding. The following table shows the reality:

| Codec | Max Bitrate | Lossless? | Open Source? | Linux Native? |
| --- | --- | --- | --- | --- |
| SBC | 345 kbps | No | Yes | Yes |
| AAC | 320 kbps | No | No | Partial |
| aptX HD | 576 kbps | No | No | No |
| LDAC | 990 kbps | No | No | Partial (PipeWire) |
| LC3 | 320 kbps | No | Yes | Partial |
| AetherCodec | 1,500 kbps | Near-Lossless | YES | YES |

## 2.2 Why No Lossless Codec Exists

- Bluetooth EDR bandwidth ceiling sits at approximately 1,000–1,500 kbps practical throughput for audio streams.

- Hi-Res raw audio at 24-bit/192kHz requires 9,216 kbps — a 6x gap that cannot be bridged without compression.

- At 24-bit/96kHz, raw bandwidth is 4,608 kbps — still 3x beyond BT EDR, but achievable with a smart 3x lossless compressor.

- No vendor has prioritized lossless because it requires receiver-side firmware changes, which breaks backward compatibility with existing headsets.

## 2.3 The Linux & Open Source Gap

- LDAC on Linux requires PipeWire with non-trivial configuration and has no first-party support from Sony.

- aptX and aptX HD are entirely closed; no Linux implementation exists without Qualcomm licensing.

- There is no open, documented, community-maintained high-quality Bluetooth audio codec designed for Linux-first deployment.

# 3. Goals & Non-Goals

## 3.1 Goals

- Primary: Stream 24-bit/96kHz audio from Laptop A to Laptop B over Bluetooth with near-lossless fidelity.

- Primary: Implement two codec modes: near-lossless (LPC/FLAC-based) and high-quality perceptual (MDCT-based), selectable at runtime.

- Primary: Integrate as a system audio sink on Linux via PipeWire so the codec appears in standard audio settings.

- Primary: Write entire codebase in C for portability and performance.

- Primary: Use zero external paid dependencies; all libraries must be free and open-source.

- Primary: Provide a working end-to-end demo on 2 laptops within 6 weeks.

- Secondary: Beat LDAC's 990 kbps in perceived quality at the same bitrate through better psychoacoustic modeling.

- Secondary: Publish complete codec specification openly for community extension.

- Secondary: Lay architectural groundwork for future firmware port to Bluetooth headset chips.

## 3.2 Non-Goals (This Phase)

- True lossless streaming at 24-bit/192kHz — physically impossible within BT EDR bandwidth constraints.

- Commercial Bluetooth headset support — requires firmware modification of headset chips (future phase).

- Windows audio driver integration — deferred to Phase 2.

- Mobile (Android/iOS) support — deferred.

- Hardware DSP acceleration — software-only for now.

# 4. Technical Architecture

## 4.1 System Overview

AetherCodec operates as a full-duplex audio streaming pipeline across two layers: a transport layer built on Bluetooth RFCOMM sockets, and a codec layer implementing compression and decompression in real time.

End-to-End Signal Flow

[Hi-Res FLAC/WAV File on Laptop A]

↓

[AetherCodec Encoder — C]

• Mode selection (Near-Lossless / Perceptual HQ)

• Frame segmentation (1024 or 2048 samples/frame)

• Compression (LPC+Rice OR MDCT+Psychoacoustic)

• Packetization with sequence numbers + timestamps

↓

[Bluetooth RFCOMM Socket — L2CAP transport]

↓ (wireless link)

[AetherCodec Decoder — C]

• Packet reorder + error concealment

• Decompression

• PCM reconstruction at 24-bit/96kHz

↓

[PipeWire Virtual Sink on Laptop B]

↓

[3.5mm Headphone Jack DAC → Headphones]

## 4.2 Transport Layer — RFCOMM over Bluetooth

The transport layer bypasses the standard A2DP profile entirely and uses raw RFCOMM sockets. This gives full control over packetization, timing, and bitrate — something A2DP does not allow.

- Protocol: Bluetooth Classic (BR/EDR) RFCOMM over L2CAP

- Practical bandwidth: 1,000–1,500 kbps sustained

- Packet structure: sequence number + timestamp + mode + sample rate + payload

- Error handling: sequence-based packet loss detection with interpolative concealment

- Jitter buffer: 20–40ms adaptive buffer on receiver side

## 4.3 Codec Layer — Two Modes

### Mode 1: Near-Lossless (LPC + Rice Coding)

Modeled on FLAC's architecture. Uses Linear Predictive Coding to predict each audio sample from previous samples. Only the residuals (prediction errors) are stored. Residuals are then entropy-coded with Rice coding for maximum compression without any information loss.

| Target Bitrate | 1,200–1,500 kbps |
| --- | --- |
| Sample Rate | 24-bit / 96kHz (fallback to 48kHz if link degrades) |
| Compression Ratio | ~3x (lossless) |
| LPC Order | 8–32 (adaptive per frame) |
| Frame Size | 2048 samples (~21ms at 96kHz) |
| Entropy Coding | Rice coding with adaptive parameter selection |
| Quality Priority | Accuracy over bitrate — never introduces distortion |

### Mode 2: High Quality Perceptual (MDCT + Psychoacoustics)

Based on the same foundational transforms used in AAC and Opus. Uses the Modified Discrete Cosine Transform to convert audio to the frequency domain, then applies a psychoacoustic masking model to identify frequencies the human ear cannot perceive in the presence of louder nearby frequencies. Only masked frequencies are quantized aggressively — audible content is preserved at high precision.

| Target Bitrate | 900–1,100 kbps |
| --- | --- |
| Sample Rate | 24-bit / 96kHz |
| MDCT Frame Size | 1024 samples (~10.7ms at 96kHz) |
| Frequency Bands | 64 Bark-scale bands |
| Psychoacoustic Model | Simultaneous + temporal masking curves |
| Quantization | Non-uniform, per-band based on masking threshold |
| Quality Priority | Perceived fidelity — transparent at bitrate target |

## 4.4 Adaptive Bitrate Engine

The encoder continuously monitors Bluetooth link quality metrics and switches codec mode or sample rate to maintain the best possible audio quality without dropouts.

| Signal Condition | RSSI | Packet Loss | Action |
| --- | --- | --- | --- |
| Excellent | > -65 dBm | < 1% | Near-Lossless 96kHz @ ~1,400 kbps |
| Good | -65 to -75 | < 3% | Near-Lossless 48kHz @ ~800 kbps |
| Moderate | -75 to -80 | < 8% | Perceptual HQ 96kHz @ ~1,000 kbps |
| Weak | < -80 dBm | > 8% | Perceptual HQ 48kHz @ ~600 kbps |

## 4.5 OS Integration — Linux PipeWire Sink

On Laptop B, AetherCodec registers itself as a virtual audio sink in PipeWire. This means any application on Laptop A can select 'AetherCodec Output' from standard system audio settings — no special software required on the sender side beyond the encoder daemon.

- PipeWire module: pw_stream registered as a capture sink

- Device name exposed: 'AetherCodec Hi-Res BT'

- Supported formats: PCM 24-bit / 96kHz / Stereo

- ALSA fallback: virtual ALSA device for systems without PipeWire

# 5. AetherCodec Specification (v1.0)

## 5.1 Packet Format

AetherPacket Structure (Binary)

Offset Size Field

──────────────────────────────────────────────

0 4 bytes Magic: 0xAE7HC0DE

4 4 bytes Sequence number (uint32)

8 4 bytes Timestamp (microseconds, uint32)

12 1 byte Mode: 0=Near-Lossless, 1=Perceptual

13 1 byte Sample rate: 0=44.1k 1=48k 2=88.2k 3=96k

14 1 byte Bit depth: 16, 24, or 32

15 1 byte Channels: 1 or 2

16 2 bytes Payload size in bytes (uint16)

18 2 bytes CRC-16 of header

20 N bytes Compressed audio payload

20+N 4 bytes CRC-32 of payload

## 5.2 Supported Configurations

| Parameter | Supported Values | Recommended |
| --- | --- | --- |
| Sample Rate | 44.1kHz, 48kHz, 88.2kHz, 96kHz | 96kHz |
| Bit Depth | 16-bit, 24-bit | 24-bit |
| Channels | Stereo (2ch), Mono (1ch) | Stereo |
| Frame Size | 512, 1024, 2048 samples | 2048 (NL) / 1024 (HQ) |
| Max Bitrate | Up to 1,500 kbps | 1,200 kbps |
| Min Bitrate | 300 kbps (fallback) | N/A |
| Latency | 20ms – 80ms end-to-end | ~40ms |

# 6. Implementation Plan

## 6.1 Phase Breakdown

Phase 1 — Transport Foundation (Week 1–2)

→ RFCOMM socket setup on both laptops (C, BlueZ)

→ Raw PCM streaming without compression

→ Packet structure implementation with sequence numbers

→ Basic jitter buffer on receiver

→ Latency measurement and logging

→ Goal: Prove the pipe works. Measure baseline latency.

Phase 2 — Near-Lossless Codec (Week 2–3)

→ LPC encoder (Levinson-Durbin algorithm)

→ Rice entropy coder / decoder

→ Frame-level encode/decode loop

→ Integration with transport layer

→ Spectral analysis comparison: original vs decoded

→ Goal: Lossless round-trip at 24-bit/96kHz.

Phase 3 — Perceptual HQ Codec (Week 3–4)

→ MDCT implementation (or leverage FFTW3, free)

→ Bark-scale frequency band mapping

→ Simultaneous masking threshold computation

→ Per-band quantization

→ Huffman or arithmetic entropy stage

→ Goal: Transparent quality at ~1,000 kbps.

Phase 4 — OS Integration (Week 4–5)

→ PipeWire virtual sink module (Laptop B)

→ PipeWire source capture module (Laptop A sender)

→ System tray / CLI mode switcher

→ ALSA fallback plugin

→ Goal: Stream from any app on Laptop A, play on Laptop B headphones.

Phase 5 — Adaptive Bitrate + Polish (Week 5–6)

→ RSSI + packet loss monitoring

→ Adaptive mode switching logic

→ Graceful degradation without audible glitches

→ Demo preparation + spectral analysis report

→ Goal: Robust demo under varying Bluetooth conditions.

# 7. Test & Validation Setup

## 7.1 Hardware Configuration

| Laptop A (Sender) | Any Linux laptop with Bluetooth 4.0+ and audio playback capability |
| --- | --- |
| Laptop B (Receiver) | Any Linux laptop with Bluetooth 4.0+ and 3.5mm headphone jack (line-out) |
| Cable | 3.5mm stereo cable: Laptop B headphone jack → Laptop A line-in (for loopback analysis) |
| Headphones | Any 3.5mm wired headphones connected to Laptop B |
| OS | Ubuntu 22.04+ or Arch Linux with PipeWire |

## 7.2 Why This Test Rig Is Valid

Laptop B's headphone jack routes decoded audio through the laptop's internal DAC — a real digital-to-analog conversion stage. This means the quality ceiling is determined entirely by the codec, not by a headset chip. This is actually a cleaner measurement environment than testing with consumer Bluetooth headphones.

- Use line-in (not mic-in) on Laptop A for loopback capture — mic-in adds noise floor and AGC distortion.

- If only mic-in is available, use a USB audio interface as line-in input for clean measurement.

- Record the loopback and run spectral FFT comparison between original file and decoded output.

## 7.3 Quality Metrics

| Metric | Tool | Target (Near-Lossless) | Target (Perceptual HQ) |
| --- | --- | --- | --- |
| THD+N | REW / sox spectrum | < -90 dB | < -80 dB |
| Frequency Response | sox / Audacity | ±0.1 dB to 48kHz | ±0.5 dB to 48kHz |
| Latency | Custom timestamp log | < 60ms | < 50ms |
| Packet Loss | Sequence counter | < 0.1% | < 0.5% |
| Bitrate | Custom logger | ~1,400 kbps | ~1,000 kbps |
| PESQ / MUSHRA | PESQ tool (free) | > 4.0 MOS | > 3.8 MOS |

# 8. Differentiation from Existing Codecs

| Feature | LDAC | aptX HD | LC3 | AetherCodec |
| --- | --- | --- | --- | --- |
| Open Source | No | No | Spec only | Full source |
| Lossless Mode | No | No | No | Yes (NL mode) |
| Linux Native | Partial | No | Partial | Yes (PipeWire) |
| Published Spec | No | No | Yes | Yes |
| Adaptive Lossless | No | No | No | Yes |
| Max Effective Quality | 24/96 lossy | 24/48 lossy | 24/48 lossy | 24/96 near-lossless |
| Tunable Psychoacoustics | No | No | No | Yes (open model) |
| Free to Implement | No | No | Limited | Yes (MIT/Apache) |

## 8.1 Unique Value Propositions

- 1: Lossless-first design: degrades to perceptual only when forced by bandwidth, never by default.

- 2: Fully open specification: every algorithm, masking curve, and packet format is documented publicly.

- 3: Linux-native PipeWire integration: no configuration hacks, appears as a standard audio device.

- 4: Tunable psychoacoustic model: community can improve masking curves without forking the codec.

- 5: Designed for real hardware: architecture is explicitly planned for firmware port to BT headset chips in Phase 3.

# 9. Future Roadmap

## 9.1 Phase 3 — Headset Firmware (Post-Demo)

To stream AetherCodec audio to a real Bluetooth headset, the decoder must run on the headset's processor. This requires either a development kit from the chip vendor or an open-hardware headset.

- Qualcomm QCC5100 series dev kit — supports custom codec plugins via their CSR SDK

- ESP32-based open headphone project — microcontroller powerful enough for LPC decode at 44.1kHz

- Nordic nRF5340 — BLE Audio chip with enough processing for lightweight decode

## 9.2 Phase 4 — Windows Support

- Implement as a Windows WASAPI exclusive mode virtual device

- Package as a driver using the Windows Driver Kit (WDK) — free

- Register codec in Windows Bluetooth audio stack via custom SDP service record

## 9.3 Phase 5 — BLE Audio Migration

- BLE Audio (Bluetooth 5.2+) with LC3 base layer + AetherCodec transport

- Isochronous channels provide more stable bandwidth for audio vs EDR

- Multi-stream audio (left/right channel independence) possible on BLE Audio

# 10. Risks & Mitigations

| Risk | Likelihood | Impact | Mitigation |
| --- | --- | --- | --- |
| BT EDR bandwidth insufficient for 24/96 NL mode | Medium | High | Fall back to 24/48 NL mode automatically; still beats LDAC |
| PipeWire API changes break sink module | Low | Medium | Pin PipeWire version; test on LTS Ubuntu |
| MDCT psychoacoustic model sounds worse than expected | Medium | Medium | Start with well-documented Bark masking model; tune iteratively |
| Laptop Bluetooth chipset limits RFCOMM throughput | Medium | High | Benchmark early (Week 1); adjust target bitrate accordingly |
| Packet loss too high for NL mode in real conditions | Low | Medium | Adaptive bitrate fallback; interpolative error concealment |
| Latency too high for practical use | Low | Low | Tune jitter buffer; demo is music listening, not real-time monitoring |

# 11. Dependencies & Tools

| Dependency | Purpose | License | Cost |
| --- | --- | --- | --- |
| BlueZ | Linux Bluetooth stack + RFCOMM API | GPL v2 | Free |
| PipeWire | Audio routing + virtual sink API | MIT/LGPL | Free |
| FFTW3 | Fast FFT for MDCT (optional) | GPL v2 | Free |
| SoX | Audio analysis + spectral comparison | LGPL | Free |
| Audacity | Waveform + spectrum visualization | GPL | Free |
| GCC / Clang | C compiler | GPL / Apache | Free |
| CMake | Build system | BSD | Free |
| PESQ tool | Perceptual quality scoring | ITU (free) | Free |
| git | Version control | GPL | Free |

# 12. Open Questions

- What is the actual sustained RFCOMM throughput on our specific laptop Bluetooth chipsets? (Measure in Week 1)

- Is FFTW3 fast enough for real-time MDCT at 96kHz on a laptop CPU, or do we need a custom FFT?

- Should we target PipeWire exclusively or also implement an ALSA plugin for broader Linux compatibility?

- What is the minimum LPC order that gives acceptable compression at 96kHz vs 48kHz content?

- Can we use the Linux kernel's existing BT driver stack for throughput beyond standard RFCOMM limits?

- For the future headset port: which open-hardware BT headset project is most active and well-documented?

# 13. Glossary

| Term | Definition |
| --- | --- |
| A2DP | Advanced Audio Distribution Profile — standard BT audio streaming protocol |
| RFCOMM | Radio Frequency Communication — serial cable emulation over Bluetooth |
| LPC | Linear Predictive Coding — predicts audio samples from previous samples |
| MDCT | Modified Discrete Cosine Transform — frequency-domain transform used in AAC/Opus |
| Psychoacoustics | Science of how humans perceive sound; used to identify inaudible frequencies |
| Masking | When a loud sound prevents the ear from hearing a quieter nearby sound |
| Rice Coding | Entropy coding optimized for small integers; used in FLAC |
| PipeWire | Modern Linux audio/video routing daemon replacing PulseAudio |
| EDR | Enhanced Data Rate — Bluetooth Classic mode with up to 3 Mbps theoretical |
| BLE Audio | Bluetooth Low Energy Audio — new standard in BT 5.2 using LC3 codec |
| DAC | Digital-to-Analog Converter — converts digital audio to analog signal for headphones |
| RSSI | Received Signal Strength Indicator — measures Bluetooth link quality |
| THD+N | Total Harmonic Distortion + Noise — measures audio output fidelity |
| PESQ | Perceptual Evaluation of Speech Quality — ITU standard for audio quality scoring |